BUFR TABLES D

(Version 7 - originating center 247)

| TABLE REFERENCE |  |  |  | TABLE REFERENCES |  |  | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  | X | Y |  |  |  |  |
|  |  |  |  |  |  |  | General information about radar product |
| 3 | 1 |  | 192 | 3 | 1 | 11 | Date of observation |
|  |  |  |  | 3 | 1 | 12 | Time of observation |
|  |  |  |  | 3 | 1 | 23 | Latitude/longitude of top left corner of the image |
|  |  |  |  | 3 | 1 | 23 | Latitude/longitude of top right corner of the image |
|  |  |  |  | 3 | 1 | 23 | Latitude/longitude of bottom right corner of the image |
|  |  |  |  | 3 | 1 | 23 | Latitude/longitude of bottom left corner of the image |
|  |  |  |  | 0 | 29 | 201 | Projection type |
|  |  |  |  | 0 | 5 | 2 | Latitude of radar |
|  |  |  |  | 0 | 6 | 2 | Longitude of radar |
|  |  |  |  | 0 | 5 | 33 | Pixel size on horizontal between columns |
|  |  |  |  | 0 | 6 | 33 | Pixel size on horizontal between rows |
|  |  |  |  | 0 | 30 | 21 | Number of pixels per row |
|  |  |  |  | 0 | 30 | 22 | Number of pixels per column |
|  |  |  |  |  |  |  | Projection information |
| 3 | 1 |  | 193 | 0 | 29 | 199 | Semi-major axis of rotation ellipsoid |
|  |  |  |  | 0 | 29 | 200 | Semi-minor axis of rotation ellipsoid |
|  |  |  |  | 0 | 29 | 193 | Longitude Origin |
|  |  |  |  | 0 | 29 | 194 | Latitude Origin |
|  |  |  |  | 0 | 29 | 195 | X-coordinate of the upper left corner of the upper left pixel of the image |
|  |  |  |  | 0 | 29 | 196 | Y-coordinate of the upper left corner of the upper left pixel of the image |
|  |  |  |  | 0 | 29 | 197 | 1st Standard Parallel |
|  |  |  |  | 0 | 29 | 198 | 2nd Standard Parallel |
|  |  |  |  |  |  |  | General information about radar product (high resolution) |
| 3 | 1 |  | 194 | 3 | 1 | 11 | Date of observation |
|  |  |  |  | 3 | 1 | 12 | Time of observation |
|  |  |  |  | 3 | 1 | 21 | Latitude/longitude of top left corner of the image |
|  |  |  |  | 3 | 1 | 21 | Latitude/longitude of top right corner of the image |
|  |  |  |  | 3 | 1 | 21 | Latitude/longitude of bottom right corner of the image |
|  |  |  |  | 3 | 1 | 21 | Latitude/longitude of bottom left corner of the image |
|  |  |  |  | 0 | 29 | 201 | Projection type |
|  |  |  |  | 0 | 5 | 1 | Latitude of radar |
|  |  |  |  | 0 | 6 | 1 | Longitude of radar |
|  |  |  |  | 0 | 7 | 1 | Height of radar |
|  |  |  |  | 0 | 5 | 33 | Pixel size on horizontal between columns |
|  |  |  |  | 0 | 6 | 33 | Pixel size on horizontal between rows |
|  |  |  |  | 0 | 30 | 21 | Number of pixels per row |
|  |  |  |  | 0 | 30 | 22 | Number of pixels per column |
|  |  |  |  |  |  |  | Heights of side view |
| 3 | 13 |  | 192 | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 10 | 7 | Height of side view |
|  |  |  |  |  |  |  | 4 bit per pixel radar images (top view) |
| 3 | 21 |  | 192 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  |  |  |  | 8 bit per pixel radar images (top view) |
| 3 | 21 |  | 193 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  |  |  |  | 4 bit per pixel radar image (north-south view) |
| 3 | 21 |  | 194 | 1 | 10 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  |  |  |  | 8 bit per pixel radar image (north-south view) |
| 3 | 21 |  | 195 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  |  |  |  | 4 bit per pixel radar image (east-west view) |
| 3 | 21 |  | 196 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  |  |  |  | 8 bit per pixel radar image (east-west view) |
| 3 | 21 |  | 197 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  |  |  |  | Rain accumulation product |
| 3 | 21 |  | 198 | 1 | 3 | 0 | Delayed replication of 3 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 13 | 16 | Precipitable water |
|  |  |  |  |  |  |  | Polar reflectivity data for a given elevation |
| 3 | 21 |  | 199 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 3 | 1 | 13 | Time of scan start |
|  |  |  |  | 0 | 2 | 134 | Antenna beam azimuth |
|  |  |  |  | 0 | 2 | 135 | Antenna elevation |
|  |  |  |  | 3 | 21 | 6 | Integration characteristics |
|  |  |  |  | 0 | 21 | 201 | Range-bin size |
|  |  |  |  | 0 | 21 | 202 | Azimuthal resolution |
|  |  |  |  | 0 | 2 | 193 | Antenna rotation direction |
|  |  |  |  | 0 | 30 | 194 | Number of bins along the radial |
|  |  |  |  | 0 | 30 | 195 | Number of azimuths |
|  |  |  |  | 3 | 21 | 193 | Compressed pixmap |
|  |  |  |  |  |  |  | Compressed rainfall accumulation data |
| 3 | 21 |  | 200 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data replication factor |
|  |  |  |  | 0 | 13 | 11 | Total precipitation |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 13 | 11 | Total precipitation |
|  |  |  |  |  |  |  | Polar radial wind data for a given elevation |
| 3 | 21 |  | 201 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 3 | 1 | 13 | Time of scan start |
|  |  |  |  | 0 | 2 | 134 | Antenna beam azimuth |
|  |  |  |  | 0 | 2 | 135 | Antenna elevation |
|  |  |  |  | 3 | 21 | 6 | Integration characteristics |
|  |  |  |  | 0 | 21 | 201 | Range-bin size |
|  |  |  |  | 0 | 21 | 202 | Azimuthal resolution |
|  |  |  |  | 0 | 2 | 193 | Antenna rotation direction |
|  |  |  |  | 0 | 30 | 194 | Number of bins along the radial |
|  |  |  |  | 0 | 30 | 195 | Number of azimuths |
|  |  |  |  | 3 | 21 | 202 | Compressed radial wind data array |
|  |  |  |  |  |  |  | Compressed radial wind data array |
| 3 | 21 |  | 202 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  |  | 0 | 5 | 31 | Row number |
|  |  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data replication factor |
|  |  |  |  | 0 | 21 | 14 | Doppler mean velocity |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 21 | 14 | Doppler mean velocity |
|  |  |  |  |  |  |  | ODIM polar volume mandatory content |
| 3 | 21 |  | 203 | 1 | 12 | 0 | Delayed replication of 12 descriptors |
|  |  |  |  | 0 | 31 | 1 | Number of scans |
|  |  |  |  | 3 | 21 | 205 | Time period (/dataset?/what/startdate, /dataset?/what/starttime, /dataset?/what/enddate and /dataset?/what/endtime) |
|  |  |  |  | 0 | 30 | 196 | Type of product (/dataset?/what/product) |
|  |  |  |  | 0 | 2 | 135 | Antenna elevation (/dataset?/where/elangle) |
|  |  |  |  | 0 | 30 | 194 | Number of bins along the radial (/dataset?/where/nbins) |
|  |  |  |  | 0 | 21 | 201 | Range-bin size (/dataset?/where/rscale) |
|  |  |  |  | 0 | 21 | 203 | Range-bin offset (/dataset?/where/rstart) |
|  |  |  |  | 0 | 30 | 195 | Number of azimuths (/dataset?/where/nrays) |
|  |  |  |  | 0 | 2 | 134 | Antenna beam azimuth (of first ray) (/dataset?/where/a1gate) |
|  |  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Number of parameters |
|  |  |  |  | 0 | 30 | 196 | Type of product (/dataset?/data?/what/quantity) |
|  |  |  |  | 3 | 21 | 206 | ODIM compressed array |
|  |  |  |  |  |  |  | ODIM additional station identifier |
| 3 | 21 |  | 204 | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  |  | 0 | 1 | 192 | Type of station identifier (RAD, ORG, PLC, CTY or CMT) |
|  |  |  |  | 0 | 1 | 193 | Station identifier |
|  |  |  |  |  |  |  | ODIM times stamps |
| 3 | 21 |  | 205 | 1 | 2 | 2 | Replication of 2 descriptors (start time stamp and end time stamp) |
|  |  |  |  | 3 | 1 | 11 | Date (year, month, day) |
|  |  |  |  | 3 | 1 | 13 | Time (hour, minute, second) |
|  |  |  |  |  |  |  | ODIM compressed array |
| 3 | 21 |  | 206 | 0 | 30 | 197 | Compression method |
|  |  |  |  | 1 | 3 | 0 | Compressed array of IEEE double floats |
|  |  |  |  | 0 | 31 | 2 | Number of chunks |
|  |  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  |  | 0 | 31 | 2 | Size of chunk |
|  |  |  |  | 0 | 30 | 198 | Byte element of a compressed array |
|  |  |  |  |  |  |  | List of radars included in a composite |
| 3 | 21 |  | 250 | 1 | 4 | 0 | Delayed replication of 4 descriptors |
|  |  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  |  | 0 | 1 | 1 | WMO block number |
|  |  |  |  | 0 | 1 | 2 | WMO station number |
|  |  |  |  | 0 | 31 | 31 | Data present indicator |
|  |  |  |  | 0 | 33 | 3 | Quality information |